| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Bulletin No BE-001/76 „OGAR” | All issued Technical Service Manuals for gliders from B-599 to B-603, B-605 and B-606 | Changes in Technical Service Manual of SZD-45A „OGAR” motoglider | To be introduced currently for removing noticed faulty in the manual |  |
| 2 | Bulletin No BE-002/76 „OGAR” | All Technical Service Manuals issued in 1976 Technical Service Manuals for the „Limbach” SL 1700 EC Engine | Changes in Technical Service Manual of SZD-45A “Ogar” motoglider and Technical Service Manual for “Limbach” SL 1700 EC Engine | To be introduced currently |  |
| 3 | Bulletin No BE-003/76 „OGAR” | All gliders with mufflers fitted acc. to the design drawing 96-00-00 /fact. Nos from B-598 to B-663/ | Mufflers fitting | To be introduced immediately |  |
| 4 | Bulletin No BE-004/76 “OGAR” | All gliders from B-598 to B-657 | Strengthening of the rudder lower fitting | On the gliders having faulty screw fixing – to be introduced immediately. On the other gliders – during the nearest periodic maintenance |  |
| 5 | Bulletin No BE-005/76 „OGAR” | All gliders | Prolongation of overhaul time for airscrew hub set | To be introduced in contact with Authority after exceeded overhaul time |  |
| 6 | Bulletin No BJ-006/76 “OGAR” | All gliders, particularly fact. Nos from B-598 to B-775 | Life-time of fuel and oil elastic ducts on the SL 1700 EC engine | Comply allowable life-time |  |
| 7 | Bulletin No BE-007/76 “OGAR” | Gliders with fact. Nos from B-598 to B-661 | Changing the page 32 onto 32a in Flight Manual, issue June 28, 1975 | After receipt of the bulletin |  |
| 8 | Bulletin No BE-008/76 “OGAR” | All gliders with fact. Nos from B-598 to B-661 | Rubbing between the fuel pump and cylinder outer deflector | In case of rubbing – to be introduced immediately |  |
| 9 | Bulletin No BR-009/77 “OGAR” | All gliders with fact. Nos from B-598 to B-752 | Reinforcement of stabilizer rear fitting | To be introduced at the nearest inspection for airworthiness prolongation |  |

| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 10 | Bulletin No BR-010/77 “OGAR” | All gliders with fact. Nos from B-598 to B-750 | Reinforcement of vertical stabilizer riveted joint | To be introduced at the nearest periodic inspection |  |
| 11 | Bulletin No BK-011/77 “OGAR” | All gliders with fact. Nos from B-598 to B-761 | „Shimmy” vibrations of tail wheel | When the vibrations appear – to be introduced immediately or at the nearest inspection for airworthiness prolongation |  |
| 12 | Bulletin No BE-012/77 “OGAR” | Gliders with fact. Nos X-107 and from B-598 to B-606 from B-643 to B-663 from B-751 to B-775 | Extension of the airscrew hub unit operation time from 350 to 500 hours | To be introduced at the nearest periodic inspection |  |
| 13 | Bulletin No BJ-013/77 “OGAR” | - | Supplementary of Bulletins No BE-002/76, BE-003/76, BE-007/76, BJ-006/76 | When the bulletin is received |  |
| 14 | Bulletin No BE-014/78 “OGAR” | Gliders with fact. Nos X-107 and from B-598 to B-606 from B-643 to B-663 from B-751 to B-775 | Extension of the airscrew hub unit operation time from 500 to 650 hours | To be introduced at the nearest periodic inspection |  |
| 15 | Bulletin No BE-015/79 “OGAR” | All gliders up to fact. No B-775 incl. except B-598, B-645, B-646 and B-770 | Improvement of working condition of the vertical push-rod of elevator control system and inspection of this push-rod | To be introduced at the nearest maintenance /but before completing 300 flying hours/ |  |
| 16 | Bulletin No BR-016/79 “OGAR” | All gliders up to fact. No B-820 incl., except B-599, B-645, B-646, B-654 and B-770 | Checking of joint of front bottom point, fixing the fin to the fuselage beam, reinforcement and inspection of riveted in bottom portion of fin covering | To be introduced at the nearest periodic inspection /but before completing 300 flying hours/ |  |

| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 17 | Bulletin No BR-017/79 “OGAR” | All gliders up to fact. No B-820 incl. | Reinforcement of rudder upper suspension | If the local fitting crack has appeared – to be introduced immediately. In other case - to be introduced at the nearest periodic inspection. Until introducing the bulletin, the fitting should be inspected each 25 flying hours |  |
| 18 | Bulletin No BR-018/79 “OGAR” | All gliders being in operation, whose riveted fitting are the covering countersunk rivet of dia. 2,6 mm /it is the basic identifying criterion/. The replacement of rivets incl. the fact. Nos from B-598 to B-663 /in this group concerns both fitting/ and from B-751 to B-775 /concerns the upper fitting only/ | Replacement of rivets fixing upper and bottom front carrying fin fitting | In case of rivets loosening – to be introduced immediately. In other case – at the nearest inspection /but before completing 250 flying hours/ |  |
| 19 | Bulletin No BE-019/80 “OGAR” | When the damage has appeared | Inspection of the engine after contact of airscrew with ground /or other obstacle/ | When the damage has appear – introduce immediately |  |
| 20 | Bulletin No BE-020/80 “OGAR” | Depends on a case described in bulletin | Periodic inspection of rubber diaphragm condition in carburetor Stromberg – Zenith 150 CD and 150 CD-3 | When some disturbances at the engine starting or running the condition of diaphragm should be inspected. Apart from above case, the diaphragm condition should be inspected every 100 flying hours or after every 6 months after the last periodic inspection |  |
| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| 21 | Bulletin No BR-021/80 “OGAR” | All gliders up to fact. No B-775 incl., except X-107 and B-598 /which have the different outrigger fixing/ as well as B-599, B-601, B-604, B-605, B-645, B-646, B-647 and B-654, in which the outrigger fitting has replaced during repair | Replacement of the fitting fixing the outrigger of the tail wheel | In case of small blister of fuselage shell tube in place of the outrigger end support /deflection 3mm/, the change should be introduced at the nearest periodic inspection. If, except of above mentioned blister shell tube, also the crack longwise the overlap under riveted joint has appeared – to be introduced immediately |  |
| 22 | Bulletin No BI-022/81 “OGAR” | - | List of issued bulletins for SZD-45A „OGAR” motoglider | For information |  |
| 23 | Bulletin No BE-023/81 “OGAR” | All operated SZD-45A /SZD-45/ „Ogar” motogliders | Introducing of inspection after each 600 flying hours as condition of extending overhaul life up to 1000 flying hours, periodic inspections after each 200 flying hours | Perform the inspection after extending 600 flying hours |  |
| 24 | Bulletin No BR-024/84 “OGAR“ | All gliders with fact Nos from B-598 to B-606 from B-643 to B-663 from B-751 to B-775 from B-818 to B-828 | Inspection of the muffler brackets condition | To be introduced immediately after receipt the bulletin |  |
| 25 | Bulletin No BE-025/95 “OGAR” | The gliders with fact. Nos B-649, B-764, B-767 and other, which flown more than 1000 flying hours | Allowance for operation the SZD-45A „OGAR” motogliders of fact. Nos B-649, B-764, B-767 or other which have completed 1000 flying hours |  |  |
| 26 | Bulletin No BE/R-026/96 “OGAR” | All SZD-45A „OGAR” motogliders, which flown minimum 600 flying hours | Rules for SZD-45A „OGAR” motoglider service life extension up to 2000 flying hours | Mandatory |  |