| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Bulletin of changes No 1 SZD-9bis “Bocian” /1959/ | The first series of A, B and C type | Page: 1 Strengthened pedal structure at rear pilot’s seat. Page: 2 Strengthened undercarriage housing. Page: 3 Strengthened air brake control lever at front and rear seat. Page: 4 Strengthened frame of front canopy. Page: 5 Concerns the disconnection of rudder control cable guides on frame No 10. Page: 6 Concerns the pedal adjustment at front and rear seat. | to be introduced during the main overhaul to be introduced during the technical inspection |  |
| 2. | Bulletin of changes No 2 SZD-9bis “Bocian” /1960/ | Gliders of series A, B and C | Possibility of wheel replacement  300x125 into  300x135 | Acc. to User’s option during the main overhaul |  |
| 3. | Bulletin of changes No 30 SZD-9bis “Bocian” /SZD-22A, B, C Mucha Std/ /1961/ | All gliders on which the bending of push-rod end appeared | Page: 1 and 2 Adjusted fork end of push-rod actuating the air brake arm is to be modified to avoid its bending. Page: 3 and 4 Change of towing-hook cable length between frames 2 and 3. | To be introduced during the operation | Pages 1 and 2 of Bulletin common with SZD-22 A, B and C. |
| 4. | Bulletin of changes No 41 SZD-9bis “Bocian” /1964/ | All gliders of series: A, B, C and D. | Change “A”: Reaming of rear fittings of wing to fuselage connection /in case of troubles in assembling/ Change “B”: Concerns the canopy hinges /incorrect locking, worn/ | To be introduced in main overhaul | The sketches Nos I to IV as enclosures |
| 5. | Bulletin of changes No 46 /1965/ | All gliders of series: 1A, 1B, 1C and 1D built up to 1965 | Change of control stick | To be introduced up to 1967-03-31 | For SZD-9bis page 5 of this Bulletin is obligatory |
| 6. | Bulletin of changes No 48 /1965/ | All gliders built up to 1963 incl. having the pedals of tubes construction | Concerns pedals at rear pilot’s seat | To be introduced during the operation |  |
|  |  | All gliders | Concerns the rear pilot’s seat and the front bolt of stabilizer fitting /on fuselage/ |  |  |
| 7. | Bulletin of changes No 50 SZD-9bis “Bocian 1D” /1964/ | All gliders of “Bocian 1D” type | Supplement to Flight Manual Annex No 1 | To be introduced immediately | Page 1 to 12 |
| 8. | Bulletin of changes No 51 SZD-9bis “Bocian” /1965/ | All gliders of “Bocian” type built up to 1963 incl. | Ref. the damage of rudder hinges when glider operated on rough airfields /inspection and replacement of hinges/ | Acc. to User’s opinion |  |

| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 9. | Bulletin of changes No 58 SZD-9bis “Bocian” /1967/ | - Gliders “Bocian 1D” produced by WSK-Krosno up to March, 1968 incl. - All gliders without the tube pass-by built up to March, 1968 incl. | Ref: 1. Strengthening of structure near the towing hook 2. Incorporation of tube pass-by for hook control cable | In main overhaul or to March, 1968 incl. |  |
| 10. | Bulletin of changes No 10 SZD-9bis 1E “Bocian” /1968/ | Gliders “Bocian 1E” operated in Poland | Ref: Change of pages in Technical Service Manual and Schedule of Periodic Inspection | To be introduced immediately | Enclosures Copies of pages 0-2, 2-3, 2-4, Fig. 13a and 19a Concerns only Polish Club |
| 11. | Bulletin No BR-001/70 SZD-9 bis “Bocian” | - Gliders “Bocian” – 1C, 1D, 1E - Gliders produced in WSK-Krosno Fact. No 779 to 879 incl. | Repair of cracks of plywood covering on fuselage rear part between frame no 24 and fin spar | - To be introduced during operation - To be introduced immediately | Gliders “Bocian 1E” have this change incorporated beginning from Fact. No P-530 incl. |
| 12. | Bulletin No BR-002/72 “Bocian 1E” | Only glider of Fact. No P-514 | - Removing of play on upper hinge of rudder - Improvement of air brake control system /removed the cause of bending the fork-end of push-rod/ | Only in overhaul of P-514 glider |  |
| 13. | Bulletin No BR-003/72 “Bocian 1E” | as above | Framework repair instruction of SZD-9 “Bocian 1E” glider central fuselage portion | as above |  |
| 14. | Bulletin No BE-004/72 | All gliders “Bocian 1E” operated by G.S.T. /East Germany/ | Ref.: Change of flight limitations of SZD-9 bis 1E “Bocian” gliders for G.S.T. /East Germany/. Replacement of pages in Flight Manual /2-1, 2-2, 2-3 and 2-4/. |  |  |
| 15. | Bulletin No BK-005/73 “Bocian 1E” | All gliders “Bocian 1E” in operation, repair and under production | Ref.: Replacement of screws fastening the wheel axle in the undercarriage fork. | To be introduced immediately |  |
| 16. | Bulletin No BE-06/75 “Bocian” | All gliders of 1C and 1D versions and gliders “Bocian 1E” up to Fact. no 716 incl. | New page 3-4a for Technical Description for version SZD-9bis 1E and inspection of wing main fittings of versions SZD-9bis 1C and 1D. | To be introduced immediately |  |
| 17. | Bulletin No BE-07/75 “Bocian” | - Version “Bocian 1E” - Version 1C and 1D | - New page in Technical Description - Inspection of wing fitting after each 200 flying hours. | To be introduced immediately |  |
| 18. | Bulletin No BE-08/76 “Bocian 1E” | All gliders of “Bocian 1E” version up to Fact. No P-759 incl. | The rudder control cables secured to avoid the loosing of contact with pulley | In the nearest technical inspection |  |

| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 19. | Bulletin No BE-09/77 “Bocian” | All gliders “Bocian 1E” except of those having the change incorporated in production | Inspection opening in root rib of wing allowing to inspect the rear wing fittings | To be introduced immediately |  |
| 20. | Bulletin No BR-10/77 “Bocian 1E” | All gliders “Bocian 1E” up to Fact. No P-787 incl. | Fastening of back safety belts fittings at front seat /new fitting/ | To be introduced immediately |  |
| 21. | Bulletin No BE-11/77 “Bocian” | All gliders of 1C and 1D version | Inspection of wing main fittings condition on gliders of 1C and 1D versions operated in Polish Aeroclub | In case of positive inspection result the gliders can obtain 60 flying hours up to 1977-12-01 | Concern only Polish Aeroclub Cancelled by BE-027/86 “Bocian” |
| 22. | Bulletin No BE-12/78 SZD-9bis 1E “Bocian” | All gliders of “Bocian 1E” type | Replacement of push-rod fork end of elevator control system /duraluminium replaced by a steel one/ | To be introduced immediately |  |
| 23. | Bulletin No BE-13/78 SZD-9bis 1E “Bocian” | as above | Inspection of turnbuckle securing in cable control systems /cracks of securing wire/ | After each 50 flying hours or no less then after 4 months |  |
| 24. | Bulletin No BE-14/78 SZD-9bis 1E “Bocian” | as above | The additional stops in air brake control system /on the front pilot’s seat and in wing air brake box/ | To be introduced in the nearest inspection or repair |  |
| 25. | Bulletin No BE-15/79 “Bocian” | Gliders of “Bocian 1E” type from Fact. No P-462 | Extension of flying hours for SZD-9bis “Bocian 1E” gliders up to 1000 or 1200 hours depending on the 800 hours repair range | Acc. to user’s decision | Cancelled by BE-020/80 “Bocian” |
| 26. | Bulletin No BE-016/79 “Bocian” | All versions of “Bocian” glider | Special inspections after each 50 hours on main wing-fuselage fittings | Immediately and after each 50 flying hours | Cancelled by BE-020/80 “Bocian” |
| 27. | Bulletin No BE-017/9bis/80 “Bocian” | Gliders of Fact. Nos P-525, P-565, P-717, P-472, P-490, P-554, P-566, P-459 | Changes of statements of Bulletin No BE-016/79 in item 3.33. /i.e. non parelel lugs of wing fittings/ | To be introduced individually to each glider | Concern only East Germany Aeroclub Cancelled by BE-027/86 “Bocian” |
| 28. | Bulletin No BE-018/9bis/80 “Bocian” | All versions of “Bocian” glider | Replacement of rear wing main fitting /after 1200 flying hours/ | Acc. to user’s decision |  |
| 29. | Bulletin No BE-019/9bis/80 “Bocian” | All gliders SZD-9bis “Bocian 1D” | Prolongation of flying hours for SZD-9 bis “Bocian 1D” gliders up to 1200 flying hours |  | Cancelled by BE-020/80 “Bocian” |
| 30. | Bulletin No BE-020/80 “Bocian” | All gliders of SZD-9bis “Bocian 1E” and “1D” type | Prolongation of flying hours of SZD-9bis “Bocian 1D and 1E” gliders /up to 2000 flying hours when the statement of this Bulletin are introduced/ | Acc. to user’s decision |  |
| 31. | Bulletin No BE-021/81 “Bocian” | All gliders of SZD-9bis “Bocian” 1C, 1D, 1E type | Operational conditions for gliders on which the fitting cracks appeared | Acc. to user’s decision |  |
| 32. | Bulletin No BE-022/81 “Bocian” | All gliders of SZD-9bis “Bocian” 1D, 1E type | Conditions for extending the flying hours of SZD-9bis 1D and 1E “Bocian” glider up to 3000 flying hours | as above |  |

| Pos. | Bulletin No | Ref. to Fact. Nos | Contents of Bulletin | Way of introducing | Remarks |
| --- | --- | --- | --- | --- | --- |
| 33. | Bulletin No BK-023/82 “Bocian 1E” | Bocian 1E gliders, Fact. Nos P-425 to P-789 incl. | Replacement of duraluminum rivets into the steel ones in elevator control system lever on torque tube |  |  |
| 34. | Bulletin No BE-024/82 “Bocian” | All gliders of 1D and 1E type | Replacement the magnetofluorescent method of inspection of main fitting with the colour penetration one |  | Concerns Polish Aeroclub /and socialistic Countries eventually/ |
| 35. | Bulletin No BE-025/82 “Bocian 1E” | “Bocian 1E” gliders, Fact. Nos P-425 to P-789 | Inspection of condition of push-rod fork end with groove of elevator control system | To be introduced immediately when this Bulletin is obtained |  |
| 36. | Bulletin No BI-026/83 “Bocian” | All versions of “Bocian” gliders | Information on all the Bulletins for versions of “Bocian” gliders issued up-to-date. |  |  |
| 37. | Bulletin No BE-027/86 “Bocian” | All the gliders of SZD-9bis 1D and 1E “Bocian” type being in operation | Interpretation of the previously issued Bulletins for SZD-9 bis-1D Bocian 1D and SZD-9bis-1E Bocian-1E gliders and associated information and complements. | To be introduced immediately when this Bulletin is received |  |
| 38. | Bulletin No BI-028/86 “Bocian” | All the gliders of “Bocian” type | Actualization and complementing of the Enclosure to Bulletin No BI-026/83 “Bocian” | To be recognized as an information |  |
| 39. | Bulletin No BE-029/92 “Bocian” | All the gliders of “Bocian” type | Terms of operation of Bocian 1D and Bocian 1E gliders having completed 3000 flying hours | For information |  |
| 40. | Bulletin No BA-030/94 “Bocian” | All the gliders of “Bocian” type | Case of cracks in pedals fitting bracket at front seat | Obligatory when this Bulletin is obtained |  |
| 41. | Bulletin No BE-031/2003 | All produced gliders of SZD-9bis “Bocian” 1D and 1E type | Service life extension up to 4000 flying hours |  | Cancelled by Bulletin BE-032/2003 |
| 42. | Bulletin No BE-032/2003 | All produced gliders of SZD-9bis “Bocian” 1D and 1E type | Service life extension up to 4000 flying hours | Acc. to user’s decision |  |
| 43. | Bulletin No BE-033/9/2019 SZD-9bis “Bocian” | All produced gliders of SZD-9bis “Bocian” 1D and 1E type | Condtions for further glider operation Extension of service life up to 7000 flying hours | To be introduced immediately on receiving this Bulletin |  |